# Types of Errors in Measurement

## 📌 What is an Error?

In Physics, an error is the difference between the measured value (what we record) and the true value (the actual value of the quantity). Errors don’t mean carelessness — they are natural and happen in all measurements.

## 1. Systematic Errors

- Happen the same way every time.
- Caused by faulty instruments, wrong calibration, or poor measuring technique.
- Example: If a weighing scale is not set to zero, every reading will show extra weight.
👉 Systematic errors always shift results in one direction (too high or too low).

## 2. Random Errors

- Happen due to unpredictable changes.
- Caused by small changes in environment or human reaction time.
- Example: Different students starting/stopping a stopwatch at slightly different times.
👉 Random errors make results scatter around the true value.

## 3. Gross Errors (Human Errors)

- Happen due to carelessness or mistakes while measuring.
- Example: Reading a scale wrongly, writing down 56 instead of 65, or misplacing a decimal point.
👉 Gross errors can be avoided with careful work and double-checking.

## 4. Parallax Error

- A special error when the eye is not in line with the scale.
- Example: Reading the water level in a cylinder from above or below eye level.
👉 Always keep your eye straight with the marking to avoid parallax error.

## ✅ How to Reduce Errors

1. Check and calibrate instruments before use.
2. Take multiple readings and calculate the average.
3. Record carefully and double-check values.
4. Use correct scientific methods.

## 🌟 Key Idea

Errors cannot be completely avoided, but they can be reduced. Good scientists always mention possible errors to show how reliable their results are.

## 🔎 Comparison of Different Types of Errors

| Type of Error | Cause | Example | How to Reduce |
| --- | --- | --- | --- |
| Systematic Error | Faulty instruments, wrong calibration, poor method | Weighing scale not set to zero | Check and calibrate instruments |
| Random Error | Unpredictable changes, environment, human reaction time | Different stopwatch readings by students | Repeat experiments and take averages |
| Gross Error (Human) | Carelessness or mistakes | Recording 56 instead of 65, wrong decimal point | Work carefully, double-check values |
| Parallax Error | Eye not in line with scale | Reading water level in a cylinder from above | Read at eye level with the scale |

Difference between Error and Uncertainty in measurements:

| Feature | Error | Uncertainty |
| --- | --- | --- |
| Definition | Difference between measured value and the true value | Range within which the true value is expected to lie |
| Cause | Faulty instruments, wrong reading, or human mistakes | Limit of the measuring instrument or natural variations |
| Example | True length = 10.0 cm, measured = 10.4 cm → Error = 0.4 cm | Length = 10.0 cm ± 0.1 cm → True value is between 9.9 cm and 10.1 cm |
| Nature | Shows how “wrong” a measurement is | Shows how “sure” we are about a measurement |
| Key Idea | Error = mistake or difference | Uncertainty = confidence range |